PHASE 1 EQUIPMENT SPECIFICATION EON XR Virtual Reality / Artificial Intelligence Lab Ikhala TVET College ¦

| Document Reference | IKHALA-SPEC-PH1-2026 |
| --- | --- |

NOTE: This specification document is prepared in direct fulfilment of MoA Milestone 1 deliverable 1.1, which requires a signed Memorandum of Agreement accompanied by the Implementation Plan and the list of tools and equipment for the VR/AI workshop/computer lab. All specifications align with EON Reality hardware requirements and the merSETA EON XR programme objectives.

| 1 | PURPOSE & SCOPE |
| --- | --- |

This document provides a comprehensive procurement specification for all hardware, software, and network infrastructure required to establish the EON XR Virtual Reality / Artificial Intelligence Lab at Ikhala TVET College. The specification covers Phase 1 (Months 1–2) of the implementation plan, comprising infrastructure setup and software installation as defined in the merSETA EON XR Project Proposal and the executed Memorandum of Agreement (MoA DGYR26).

The lab will accommodate up to 30 students simultaneously and will serve as the primary facility for EON XR-enabled training across MERSETA-related courses in manufacturing, engineering, automotive technology, construction, and IT.

All items specified herein must meet or exceed the minimum EON Reality platform requirements as confirmed by EON Reality's Global Director of Education and Training, and must be procured by Ikhala TVET College in strict accordance with the College's internal Procurement Policies and the Public Finance Management Act (PFMA).

| 2 | LAB PHYSICAL INFRASTRUCTURE REQUIREMENTS |
| --- | --- |

## 2.1 Space & Layout

The following physical space and layout standards must be met before equipment installation commences.

| Parameter | Minimum Requirement | Recommended / Notes |
| --- | --- | --- |
| Floor Area | 60 m² | 120 m² preferred for full VR mobility zones |
| Ceiling Height | 2.7 m minimum | 3.0 m+ preferred for overhead cabling |
| Workstation Clearance | 1 m between tables (all sides) | Clear pathways are mandatory for VR movement safety |
| VR Activity Zone | Dedicated open-floor area, minimum 4 m × 4 m | Safety floor markers and padded barriers are required |
| Teacher/Facilitator Station | Front of room, full sightlines to all workstations | Elevated platform recommended |
| Storage Area | Lockable cabinet minimum 1.2 m × 0.6 m | For headsets, controllers, charging equipment |
| Electrical Points | Minimum 2 × 15A circuits per workstation cluster | UPS-protected distribution boards |
| Air Conditioning | Split-unit or centralised HVAC — minimum 18 000 BTU per 60 m² | CRITICAL: high-performance PCs generate significant heat |
| Lighting | Adjustable LED ambient lighting (500–1000 lux at desk level) | Dimming capability for immersive VR sessions |
| Network Conduits | Cat 6A conduit runs to all workstation positions | Concealed or floor-mounted cable management |

## 2.2 Electrical & Power Infrastructure

- Uninterruptible Power Supply (UPS) — minimum 10 kVA, 30-minute runtime — to protect all equipment from load-shedding (Quantity 1)

| 3 | COMPUTING HARDWARE — STUDENT WORKSTATIONS |
| --- | --- |

Thirty-one (31) high-specification desktop workstations are required: 30 for students and 1 dedicated facilitator/instructor unit. All units must be capable of running EON XR software and SteamVR with full DirectX 12 support.

## 3.1 Desktop PC Specification (Quantity: 31)

| Component | Recommended Specification | Compliance Basis |
| --- | --- | --- |
| Processor (CPU) | Intel Core i7-13700 or AMD Ryzen 7 7700X (12th/13th Gen or Zen 4) | EON XR VR Training requirement |
| RAM | 16 GB DDR4 3200 MHz (dual-channel) | Proposal spec — 16 GB minimum for XR workloads |
| Primary Storage | 1 TB NVMe SSD (M.2, PCIe Gen 4) | EON XR requires 5 GB+ free; SSD mandatory for performance |
| Graphics Card (GPU) | NVIDIA GeForce RTX 4060 or AMD Radeon RX 7600 (any xx60 variant or better) | DirectX 12 / SteamVR / EON XR mandatory; RTX enables AI features |
| Display | 24-inch Full HD (1920 × 1080), IPS panel, 75 Hz+ | Proposal spec — 24-inch 1080p standard |
| Connectivity — Video | HDMI 2.0 + DisplayPort 1.4 (dual outputs) | Required for VR headset tethering (Oculus Link) |
| Connectivity — USB | Minimum 4 × USB 3.2 Gen 1 (Type-A) + 1 × USB-C 3.2 | VR headset and peripheral connectivity |
| Connectivity — Network | Intel I225-V 2.5 GbE LAN + Wi-Fi 6 (802.11ax) card | Required for EON XR cloud content & collaboration |
| Operating System | Windows 11 Pro (64-bit) | EON XR minimum; Win 11 for long-term support |
| DirectX Support | DirectX 12 Ultimate | EON XR DX8/DX9/DX10 compliant; DX12 for SteamVR |
| Steam Application | Steam + SteamVR pre-installed | EON XR VR training prerequisite |
| Form Factor | Mid-tower with tool-less access for maintenance | Serviceability in education environment |
| Keyboard & Mouse | USB or wireless, full-size, spill-resistant | 1 set per workstation |
| Warranty | 3-year on-site next business day | Education/institutional procurement standard |

NOTE: The RTX 4060 GPU (or any xx60 variant) is the minimum recommended for sustained EON XR and SteamVR performance. It supports NVIDIA DLSS and ray tracing, ensuring the platform remains viable for the full duration of the MoA (through 31 March 2027) and beyond.

## 3.2 Facilitator / Instructor Workstation — Additional Requirements (Quantity: 1)

The instructor unit must meet all specifications in Section 3.1 and additionally include:

- Dual 24-inch monitor setup (extended display for content management and student monitoring)

- Webcam (minimum 1080p 30fps) for remote training and content recording

- Headset/microphone combination for virtual facilitation

- Minimum 32 GB RAM to support simultaneous content creation, streaming, and monitoring applications

- Software: Blender (3D asset creation), Unity (XR content development), EON XR Studio licence

| 4 | VIRTUAL REALITY HEADSETS |
| --- | --- |

VR headsets are the primary student immersion devices enabling access to the EON XR Metaverse builder and AI assistant functions. Two tiers are specified based on use case.

## 4.1 Meta Quest 3 — Primary VR Headsets (Quantity: 16)

| Attribute | Specification |
| --- | --- |
| Model | Meta Quest 3 (128 GB storage variant minimum) |
| Display | Pancake lens optics — 2064 × 2208 per eye, 90/120 Hz refresh rate |
| Processor | Snapdragon XR2 Gen 2 |
| RAM | 8 GB LPDDR5 |
| Storage | 128 GB (minimum) |
| Connectivity | Wi-Fi 6E (802.11ax), Bluetooth 5.3, USB-C 3.0 (for Oculus Link / Air Link) |
| Controllers | Touch Plus controllers included (1 pair per headset) |
| Operating Mode | Standalone (wireless) and PC-tethered (via Oculus Link cable or Air Link) |
| Battery Life | 2–3 hours active use per charge |
| Compatibility | EON XR platform, SteamVR (via Air Link / Virtual Desktop), Meta Horizon OS |
| Guardian System | Built-in boundary detection and passthrough camera for VR safety zones |
| Accessories Required (per unit) | 1 × USB-C charging cable (2 m), 1 × silicone face cover (hygiene), 1 × protective carry case |
| Charging Infrastructure | 16-port charging station or 4 × 4-port USB-C charging hubs (see Section 7) |
| Warranty | Minimum 1-year manufacturer warranty; extended care plan recommended |
| Justification | Long-term platform viability. Quest 2 is cheaper but EON Reality support lifecycle shorter; Quest 3 recommended per JEF technical assessment |

## 4.2 Oculus Link Cable — Tethered VR Operation (Quantity: 16)

One high-speed Oculus Link-compatible USB-C cable (minimum 5 m, USB 3.2 Gen 1) per headset for PC-tethered SteamVR operation where wireless bandwidth is insufficient.

| 5 | MOBILE DEVICES — AR FUNCTIONALITY |
| --- | --- |

Mobile devices extend EON XR's augmented reality capabilities using device cameras and ARKit/ARCore frameworks. Two platforms are specified.

## 5.1 iPad Air 11-inch (Wi-Fi) — iOS Devices (Quantity: 2)

| Attribute | Specification |
| --- | --- |
| Model | iPad Air 11-inch (M2 chip, latest generation) |
| Display | 11-inch Liquid Retina, 2360 × 1640, True Tone, P3 wide colour |
| Chip | Apple M2 (8-core CPU, 9-core GPU) |
| RAM | 8 GB |
| Storage | 256 GB minimum |
| OS | iPadOS 17 or latest (requires iOS 10 minimum for EON XR) |
| Connectivity | Wi-Fi 6E (802.11ax) — Wi-Fi only model specified for cost efficiency |
| Camera | 12 MP rear camera with LiDAR Scanner — enables 3D scanning and ARKit spatial mapping |
| AR Framework | ARKit (Apple) — superior AR tracking and 3D scanning capability |
| EON XR Compatibility | Full — iOS 10+, 2 GB RAM minimum (iPad Air exceeds requirements) |
| Accessories | Smart Folio case (protective), Apple Pencil 2nd gen (1 unit, for content annotation) |
| Justification | LiDAR + ARKit enables 3D scan functionality critical for EON XR content creation |

## 5.2 Android Tablets / Devices (Quantity: 13)

| Attribute | Minimum Requirement | Recommended Device |
| --- | --- | --- |
| OS | Android 5.0 Lollipop (API 21) minimum | Android 13 or later |
| OpenGL | OpenGL ES 3.0 support mandatory | OpenGL ES 3.2 |
| RAM | 3 GB minimum | 6 GB+ (e.g., Samsung Galaxy Tab A9+) |
| Storage | 32 GB minimum | 128 GB |
| Display | Minimum 8-inch screen | 10-inch+ Full HD (1920 × 1200) |
| Camera | Rear camera with autofocus | 13 MP+ for AR scanning |
| AR Framework | ARCore support required | Verified ARCore device list |
| Connectivity | Wi-Fi (802.11ac minimum) | Wi-Fi 6 preferred |
| Battery | Minimum 6 000 mAh | 7 000+ mAh |
| Recommended Model | — | Samsung Galaxy Tab A9+ (Wi-Fi, 4 GB RAM / 64 GB or 8 GB / 128 GB) |
| Accessories | Protective case (shockproof) per device | Screen protector per device |

| 6 | PROJECTION & DISPLAY EQUIPMENT |
| --- | --- |

## 6.1 3D DLP Projector (Quantity: 1)

A high-lumen 3D-capable projector is required for whole-class EON XR content display, 3D visualisation, and immersive environment demonstrations.

| Attribute | Specification |
| --- | --- |
| Model Reference | Dell 210-AGYT 4350 Full HD or equivalent specification |
| Technology | DLP (Digital Light Processing) — Active Stereo 3D capable |
| Resolution | Full HD 1920 × 1080 (native), 120 Hz for 3D Stereo |
| Brightness | Minimum 4 000 ANSI Lumens |
| 3D Support | True Active 3D Stereo (frame sequential, compatible with DLP Link glasses) |
| Throw Ratio | Short-throw or Ultra Short-throw lens preferred for lab dimensions |
| Screen Size | 100-inch diagonal minimum (motorised, front-projection) |
| Screen Type | Motorised retractable, gain optimised for 3D active stereo (gain 1.0–1.4) |
| Connectivity | HDMI 2.0, DisplayPort 1.2, VGA, USB-A (media playback) |
| Mounting | Ceiling-mounted bracket with alignment controls, safety cable, all cabling included |
| Lamp Life | Minimum 4 000 hours (eco mode) |
| EON XR Compatibility | Effective resolution per screen: 1920 × 1080 @ 120 Hz — EON 3D Visualisation certified |
| Projector PC (if integrated system) | Intel Xeon 3.0 GHz+, 32 GB DDR3+, NVIDIA Quadro P5000 GPU — for dedicated visualisation node |

| 7 | NETWORK INFRASTRUCTURE |
| --- | --- |

A robust, dedicated network is essential for EON XR cloud content synchronisation, real-time multi-user collaboration, and VR streaming. The following infrastructure must be installed and commissioned as part of Phase 1.

## 7.1 Internet Connectivity

| Parameter | Requirement | Notes |
| --- | --- | --- |
| Connection Type | Fibre-to-the-premises (FTTP) or Fibre-to-the-building (FTTB) | No ADSL or LTE as primary connection |
| Bandwidth | Minimum 100 Mbps symmetric (upload = download) | 200 Mbps+ recommended for 30 simultaneous users |
| Latency | < 20 ms to EON XR cloud servers | Critical for real-time VR collaboration |
| SLA | 99.5% uptime minimum with 4-hour fault response | Business-grade ISP SLA required |
| Redundancy | LTE/5G failover router as backup | Prevents learning disruption during fibre outages |

## 7.2 Local Area Network (LAN) — Switching

- 1 × Managed Layer 2/3 Gigabit Ethernet switch — minimum 48 ports (e.g., Cisco Catalyst 1000-48T or equivalent)

- VLAN segmentation: dedicated VLAN for lab workstations, separate VLAN for VR headsets (wireless), management VLAN

- QoS (Quality of Service) configuration: prioritise VR traffic and EON XR cloud traffic

- All switch ports to workstations: Cat 6A patch cables, minimum 1 Gbps

- 10 GbE uplink to server/router for aggregated bandwidth headroom

## 7.3 Wireless Network (Wi-Fi) — VR Headset Connectivity

| Parameter | Requirement |
| --- | --- |
| Wi-Fi Standard | Wi-Fi 6E (IEEE 802.11ax) — 6 GHz band mandatory for VR headset Air Link |
| Access Points | Minimum 2 × Wi-Fi 6E ceiling-mounted APs with coverage overlap (e.g., Ubiquiti U6-Enterprise or equivalent) |
| Throughput per AP | Minimum 4.8 Gbps aggregate (tri-band) |
| Client Density | Rated for 50+ concurrent clients per AP |
| Coverage | Full lab coverage with -65 dBm minimum signal at all points |
| Controller | Cloud or on-premises wireless controller for centralised management |
| SSID Configuration | Dedicated SSID for VR headsets (WPA3, 6 GHz), separate SSID for general devices |

## 7.4 Server & Rack Infrastructure

| Item | Quantity | Specification |
| --- | --- | --- |
| 48U Server Cabinet / Rack | 1 | 19-inch rack, lockable, ventilated, with PDU (Power Distribution Unit), cable management, and castors for mobility |
| Server Cabinet Racks (workstation patch panels) | 30 | 1U patch panel per workstation cluster; structured cabling termination |
| UPS (Rack-mounted) | 1 | Minimum 3 kVA rack-mount UPS for network equipment and server (separate from workstation UPS in Section 2.2) |
| Patch Cables (Cat 6A) | 60+ | 0.5 m and 1 m lengths; colour-coded per VLAN |
| Fibre uplink cable (SFP+) | 2 | LC-LC OM3/OM4 for switch-to-firewall uplink if applicable |

## 7.5 Device Management Infrastructure

- Mobile Device Management (MDM) Solution: Microsoft Intune, Jamf, or equivalent — mandatory for managing all 16 Meta Quest 3 headsets, 2 iPads, and 13 Android tablets centrally

- Capabilities required: remote enrolment, app push deployment (EON XR app), VPN profile distribution, remote wipe, usage monitoring

- Remote Monitoring Software: SolarWinds, PRTG, or equivalent — for workstation uptime, network bandwidth, and hardware health monitoring

- 1 × Charging Station: Multi-device charging dock accommodating minimum 16 × USB-C ports at 18W+ per port for VR headsets; additional 15-port USB-A/C for tablets and iPads

| 8 | SOFTWARE & LICENSING |
| --- | --- |

All software must be installed, configured, and tested on all workstations prior to the Phase 1 sign-off site visit (MoA Milestone 3 deliverable).

| Software / Licence | Quantity | Specification / Notes |
| --- | --- | --- |
| EON XR Platform Licence | 30+ seats | merSETA-granted licences — verify active seat allocation per campus prior installation; confirm Metaverse builder and AI assistant access |
| EON XR Mobile App | 31 devices | Deployed via MDM to all Android and iOS devices; confirm version compatibility with platform licence |
| Windows 11 Pro (64-bit) | 31 | OEM or volume licence; activated and domain-joined or Azure AD joined |
| Steam Client | 31 | Free; must be installed and accounts provisioned on all workstations |
| SteamVR | 31 | Free add-on via Steam; must be tested with Meta Quest 3 on each workstation |
| Meta Quest Link App (PC) | 31 | For wired Oculus Link; must be installed and tested pre-delivery |
| Blender (3D Modelling) | 31 | Open source — free; version 4.x; for XR content and 3D asset creation |
| Unity Hub + Unity Editor | 31 | Unity Personal/Student licence (free for educational use under threshold); for custom XR content development |
| Microsoft 365 (Office) | 31 | Existing institutional licence — confirm coverage with College IT; required for reporting and administration |
| Antivirus / Endpoint Protection | 31 | Centralised managed endpoint protection (e.g., Microsoft Defender for Endpoint or equivalent) |
| Centralised MDM Client | All devices | MDM agent installed on all managed devices (workstations, headsets, tablets) |

| 9 | VR SAFETY & LAB ACCESSORIES |
| --- | --- |

## 9.1 VR Safety Equipment

| Item | Quantity | Specification |
| --- | --- | --- |
| Floor Safety Markers | 1 set | Vinyl floor tape or rubber mat defining VR guardian boundary zones — high-visibility (yellow/black) |
| Padded Wall/Barrier Panels | As required | Foam padding or crash mat for VR activity zone walls and corners |
| VR Headset Hygiene Covers | 48+ | Silicone or disposable face covers for Meta Quest 3 (minimum 3 per headset for rotation and cleaning) |
| Headset Sanitising Kit | 2 sets | UV-C sanitising station or spray-based sanitiser (isopropyl alcohol wipes, IPA < 70%) for lens-safe cleaning |
| Lens Cleaning Cloths | 30 | Microfibre, lint-free — 1 per student per session |

## 9.2 Lab Furniture & Ergonomic Equipment

| Item | Quantity | Specification |
| --- | --- | --- |
| Student Workstation Desks | 30 | 1.2 m × 0.6 m minimum; height-adjustable 680–780 mm; cable grommets; laminate surface |
| Headset Storage/Display Shelves | 2 | Wall-mounted or free-standing; foam-padded slots for 16 headsets; cable routing |
| Round Collaboration Tables | 6 | 900 mm diameter; max 5 persons; for group XR activities per EON Reality lab layout |
| Whiteboards | 2 | Minimum 1.8 m × 1.2 m; magnetic, with marker trays |

| 10 | EQUIPMENT SUMMARY & QUANTITIES |
| --- | --- |

The following consolidated list summarises all items to be procured for Phase 1. This list constitutes the approved equipment list referenced in MoA Milestone 1 (deliverable 1.1) and Milestone 2 (deliverable 2.1 — order confirmation).

| # | Item Description | Qty | Unit | Key Specification Reference |
| --- | --- | --- | --- | --- |
| 1 | High-Spec Desktop PC (Student) ALL IN ONE DESKTOPS | 30 | Units | Section 3.1 — Intel Core i7, 16 GB RAM, 1 TB SSD, RTX 4060 |
| 2 | High-Spec Desktop PC (Instructor) | 1 | Unit | Section 3.2 — As above + dual monitor, 32 GB RAM |
| 3 | 24-inch FHD Monitor (1080p) | 31 | Units | Section 3.1 — 1920×1080, IPS, 75 Hz+ |
| 4 | Keyboard & Mouse (USB) | 31 | Sets | Section 3.1 — Spill-resistant, full-size |
| 5 | Meta Quest 3 VR Headset (128 GB) | 16 | Units | Section 4.1 — Wi-Fi 6E, USB-C, Touch Plus controllers |
| 6 | Oculus Link USB-C Cable (5 m) | 16 | Units | Section 4.2 — USB 3.2 Gen 1, 5 m minimum |
| 7 | iPad Air 11-inch Wi-Fi (256 GB) | 2 | Units | Section 5.1 — M2 chip, LiDAR, ARKit |
| 8 | Android Tablet (e.g., Samsung Tab A9+) | 13 | Units | Section 5.2 — 6 GB RAM, ARCore, Wi-Fi 6 |
| 9 | Full HD 3D DLP Projector (4000 lm) | 1 | Unit | Section 6.1 — 1920×1080, Active 3D Stereo, ceiling mount |
| 10 | 100-inch Motorised Projection Screen | 1 | Unit | Section 6.1 — 3D optimised, motorised retractable |
| 11 | Wi-Fi 6E Access Points (ceiling) | 2 | Units | Section 7.3 — Ubiquiti U6-Enterprise or equivalent |
| 12 | 48-Port Managed Gigabit Switch | 1 | Unit | Section 7.2 — Layer 3, 10 GbE uplink, VLAN/QoS |
| 13 | 48U Server Rack Cabinet | 1 | Unit | Section 7.4 — Lockable, ventilated, with PDU |
| 14 | Network Patch Panels (1U) | 30 | Units | Section 7.4 — Per workstation cluster |
| 15 | Cat 6A Structured Cabling (full run) | 1 | Lot | Section 7 — Full lab cabling, conduit, termination, testing |
| 16 | Multi-Port USB-C Charging Station (16-port) | 1 | Unit | Section 7.5 — 18W+ per port, for Quest 3 headsets |
| 17 | UPS — 10 kVA (workstation protection) | 1 | Unit | Section 2.2 — 30-minute runtime, automatic voltage regulation |
| 18 | UPS — 3 kVA rack-mount (network) | 1 | Unit | Section 7.4 — Rack-mounted, for network infrastructure |
| 19 | VR Floor Safety Markers | 1 | Set | Section 9.1 — High-visibility vinyl boundary markers |
| 20 | VR Headset Hygiene/Silicone Covers | 48 | Units | Section 9.1 — Compatible with Meta Quest 3 |
| 21 | Student Workstation Desks | 30 | Units | Section 9.2 — 1.2 m × 0.6 m, height adjustable |
| 22 | Ergonomic Student Chairs | 30 | Units | Section 9.2 — Height adjustable, castors |
| 23 | Instructor Desk + Chair | 1 | Set | Section 9.2 — 1.6 m × 0.8 m desk, high-back chair |
| 24 | Headset Storage Shelving Unit | 2 | Units | Section 9.2 — Padded slots, wall or free-standing |
| 25 | Round Collaboration Tables (5-person) | 6 | Units | Section 9.2 — 900 mm diameter |
| 26 | Lockable Equipment Storage Cabinet | 1 | Unit | Section 2.1 — For tablets, accessories |

| 11 | PROCUREMENT, COMPLIANCE & DELIVERY REQUIREMENTS |
| --- | --- |

## 11.1 Procurement Compliance

As stipulated in MoA Clause 5.4, the College is solely responsible for the procurement of all equipment. The following must be adhered to:

- All procurement must comply with the College's internal Supply Chain Management (SCM) Policy

- All procurement must comply with the Public Finance Management Act (PFMA), Act 1 of 1999

- Quotations must be obtained from a minimum of three (3) accredited suppliers per item category

- All suppliers must be registered on the Central Supplier Database (CSD)

- Preference must be given to B-BBEE compliant suppliers in line with the Preferential Procurement Policy Framework Act (PPPFA)

- Purchase orders and invoices must reference MoA number DGYR26 / ID N000100758

## 11.2 Delivery & Installation Requirements

- All hardware must be delivered to Ikhala TVET College (Zone D Ezibeleni, Gwadana Drive, 5326) with delivery notes and serial number lists

- Equipment serial numbers must be captured in the College's asset register prior to installation

- Professional installation by certified technicians is required for: network cabling, projector ceiling mount, rack installation, UPS installation, and electrical work

- All workstations must be assembled, OS installed, software configured, and tested before MoA Milestone 3 site visit

- Network infrastructure must be tested with a speed/throughput report confirming minimum 100 Mbps symmetric

- VR headsets must be enrolled in MDM, EON XR app installed, and Wi-Fi Air Link tested before handover

## 11.3 Acceptance Testing & Sign-Off

The following must be completed and documented before Milestone 3 payment

- Delivery report — signed by College principal or delegated official (MoA Clause 15.4.2)

- Asset register extract — all items listed with serial numbers

- Network test report — confirming speed, VLAN configuration, and Wi-Fi coverage

- Software installation checklist — signed off per workstation

- VR headset commissioning report — all 16 units tested with EON XR

- Site visit report — conducted by merSETA representative

- Close-out report — accompanied by proof of completion per MoA Clause 3.2 (Milestone 3 deliverable 3.2)

| 12 | DISBURSEMENT ALIGNMENT — MOA ANNEXURE A |
| --- | --- |